AeroPulse-NG — Technical Reference

Wire contracts, algorithms and verification

# 1. Hardware and Wire Formats

mode_s_decoder.rs accepts 7-byte and 14-byte Mode S frames, validates CRC-24 address/even parity, attempts single-bit repair and decodes DF17 identity, airborne position, velocity and status messages. CPR pairs are resolved inside a 10-second epoch. acars_decoder.rs parses SOH/MODE/TAIL/ACK/LABEL/BLOCK/STX/TEXT/ETX/BCS and classifies METAR, SPECI and D-ATIS reports.

# 2. Kinematics

| Component | Contract |
| --- | --- |
| Kalman filter (CV) | State [x,y,z,vx,vy,vz], ENU metres and metres per second; Joseph covariance update; acquisition warm-up and Mahalanobis gate |
| Geodesy | SiteOrigin converts WGS-84 latitude/longitude to local ENU and back |
| R*-tree | AABB index with overlap-based subtree selection, margin split and forced reinsertion |
| STCA | 5 NM / 1,000 ft thresholds, 120-second lookahead, 5-second refinement |

# 3. Weather and Defence

Weather fusion retains AWOS, ACARS and BDS-derived nodes, applies source priority and performs vector-based inverse-distance interpolation with an ISA fallback. Defence modules map 7500, 7600 and 7700; detect identity loss and silence; monitor polygon/floor/ceiling geofences; and solve constant-velocity intercept geometry.

# 4. Engine Snapshot

EngineSnapshot contains tracks, flight data blocks, STCA alerts, geofence breaches, anomaly notes, weather and EngineStatus. The Rust-to-TypeScript field names remain snake_case. The frontend converts wire feet/knots to metre and kilometre-per-hour presentation through src/lib/units.ts.

# 5. Frontend Components

| Component | Responsibility |
| --- | --- |
| RadarCanvas | 60 fps radar drawing, aircraft selection, range/bearing measurement and conflict lines |
| FlightStripTable | Arrival/departure strips and aircraft selection |
| AircraftDetailPanel | Individual aircraft data and audio-channel action |
| WeatherGridHUD | Surface and upper-air conditions with fallback state |
| AudioRoutingPanel | Two output devices, frequency selection and guarded PTT |
| SystemMetrics | Receiver status, spectrum visualisation and performance gauges |

# 6. Verification Inventory

| Suite | Expected result |
| --- | --- |
| cargo test --no-default-features | 90 Rust tests pass |
| python-sidecar pytest | 14 Python tests pass |
| npm run typecheck | TypeScript compiler exits cleanly |
| npm run build | Vite emits index.html and hud.html |