AeroPulse-NG — User Manual

Bench setup, operation and safety guidance

# 1. Purpose and Scope

This guide covers bench operation using the simulator and optional receive hardware. It is not an operational approval, an ATC procedure or an authorisation to transmit on aviation frequencies.

# 2. Installation

| Requirement | Bench recommendation |
| --- | --- |
| Host | Current laptop or desktop; 4 CPU cores and 8 GB RAM recommended |
| Receiver | Three RTL-SDR Blog V4 units for ADS-B, ACARS and VHF receive |
| Transmitter | Not required for receive-only bench testing; HackRF TX requires authorisation and controlled test conditions |
| Audio | Standard USB headset with microphone |
| Power | Powered USB 3.0 hub; do not run three receivers from a passive hub |

# 3. Start the Software

| Mode | Command |
| --- | --- |
| Full desktop | scripts/desktop.sh |
| Browser preview | npm run dev, then open http://localhost:1420/ |
| Operations display | Open http://localhost:1420/hud.html or use the Operations Display button |
| Headless | cd src-tauri && cargo run --no-default-features --bin aeropulse-headless |
| Verification | ./scripts/verify.sh |

# 4. Display 1 — Radar

Select an aircraft by clicking its symbol. Use Measure Distance, click a start point and move the pointer; double-click clears the measurement. Use Operations Display to open the second window. The radar uses kilometre range rings, metre altitude and kilometre-per-hour speed presentation.

# 5. Display 2 — Operations Workspace

On a small screen the navigation bar switches between complete sections: Flight Strips, Aircraft, Weather, Alerts, Audio and System. Only the selected section is displayed in compact mode, preventing one card from covering another. Each section has its own scroll area where required.

# 6. Aircraft Details and Audio

Select a flight strip or radar contact to open its individual details. The detail view shows identification, position, altitude, speed, heading, vertical rate, accuracy and tracking state. The Transmit to aircraft action tunes the audio panel to the assigned prototype channel; it does not bypass the transmission safety interlock.

# 7. Safety Rules

Treat all simulated values as test data until independently verified against a known source.

Receive-only testing is the default and requires no transmitter.

Never connect a transmitter to an antenna or radiate on an aviation frequency without the required Nigerian authorisation and a controlled test plan.

The software uses hold-to-talk, frequency validation, emergency inhibition and a 30-second transmission timeout; these are safeguards, not regulatory approval.

# 8. Troubleshooting

| Symptom | Action |
| --- | --- |
| Upper-air table blank | Select Weather in the compact navigation. If no reports have arrived, the panel shows aircraft estimates or an explicit waiting state. |
| Flight strip missing | Select Flight Strips and scroll inside the strip list; the list is independent of the page. |
| Audio output missing | Allow microphone permission, reconnect the USB headset and choose Primary Output again. |
| Desktop shell will not build | Install webkit2gtk-4.1 and libsoup-3.0 development packages or source scripts/webkit-env.sh. |